Chapter 5

Fire Behavior

Lesson Goal

After completing this lesson, the student shall be able to explain the science of fire behavior as it relates to recognizing stages of fire development, rapid fire behavior, and fire fighting operational safety.

Objectives

Upon successful completion of this lesson, the student shall be able to:

1. 	Explain the science of fire as it relates to energy, forms of ignition, and modes of combustion. [NFPA® 1001, 5.3.11]

2. 	Describe the impact of thermal energy on heat, temperature, and heat transfer. [NFPA® 1001, 5.3.12]

3.	Recognize the physical states of fuel. [NFPA® 1001, 5.3.10]

4.	Explain the relationship between oxygen and life safety. [NFPA® 1001, 5.3.11]

5. 	Identify the products of self-sustained chemical reactions. [NFPA® 1001, 5.3.11]

6.	Explain the factors that affect fire development. [NFPA® 1001, 5.3.11]

7.	Describe the stages of fire development. [NFPA® 1001, 5.3.11]

8.	Recognize signs, causes, and effects of rapid fire development. [NFPA® 1001, 5.3.11]

9.	Describe the methods through which fire fighting operations can influence fire behavior. [NFPA® 1001, 5.3.11, 5.3.12]

Instructor Information

This is the lesson covering fire behavior. This lesson describes the science of fire, various physical states fuel can be found in, classification of fires, and the stages of fire development. The relationship between fire behavior and fire fighting operations is also discussed.

Important instructor information is provided in shaded boxes throughout the lesson plan. Carefully review the instructor information before presenting the lesson.

Methodology

This lesson uses lecture and discussion. The level of learning is comprehension.

Table of Contents

| Section | Outline Pages | Text Pages |
| --- | --- | --- |
| Section I: Science of Fire | 5 | 208 |
| Section II: Thermal Energy (Heat) | 17 | 216 |
| Section III: Fuel | 27 | 223 |
| Section IV: Oxygen | 34 | 231 |
| Section V: Self-Sustained Chemical Reaction | 36 | 233 |
| Section VI: Stages of Fire Development | 38 | 234 |
| Section VII: Fire Behavior and Fire Fighting Operations | 59 | 250 |
| Section VIII: Summary and Review | 64 | 254 |

Audiovisuals/Handouts

- Visuals 5.1 to 5.86 (PowerPoint® Presentation)

Evaluation

- Chapter 5 Quiz

- Chapter 5 Test

Instructor Tools

Interactive Objects allow instructors to present information a piece at a time by clicking on hot spots in a larger image.

Each object is labeled in the Lesson Outline and indicated in the PowerPoint® presentation by a RED arrow in the top left corner of the image. This lets instructors know to use the mouse to explore all of the information on the slide.

After all of the information is displayed a reset arrow will appear in the bottom right corner of the image. Instructors can use this to remove the text and quiz students on content just covered or simply move on to the next slide.

To move to the slide after an interactive object, select the Click for next slide arrow on the left side of the slide.

Videos are used as discussion starters or to illustrate a concept or process discussed in the chapter.

These are labeled in the Lesson Outline and indicated in the PowerPoint® presentation by the Click image to play arrow on the left side of the slide.

Section I: Science of Fire

# Science of Fire

pp. 208-216	Objective 1 — Explain the science of fire as it relates to energy, forms of ignition, and modes of combustion.

## Science of Fire

### Fire takes variety of forms – All involve heat-producing chemical reaction between fuel and oxidizer (most common is oxygen in air)

### Understanding physical science of fire can help firefighter safety

#### Translates into practical knowledge of fire behavior

#### Be able to “read” the fire – Recognize what is happening, predict potential behavior

### Physical change

#### Substance remains chemically the same, changes in size, shape, or appearance

#### Examples

##### Water freezing – Liquid to solid

##### Boiling – Liquid to gas

### Chemical reaction

#### Substance changes from one type of matter into another

#### Examples

##### Two or more substances combining to form compounds

##### Oxidation – Combination of oxidizer with other materials; can be slow or rapid

## Energy

### Defined as capacity to perform work

### In case of heat – Defined as increasing temperature of substance

### Work occurs when

#### Force is applied to object over distance

#### Substance undergoes chemical, biological, or physical change

### Forms of

#### Potential

##### Represents amount of kinetic energy that an object can release at some point in the future

##### Fuels have certain amount of chemical potential energy before ignited

##### Different fuels release different amounts of energy over different amounts of time

#### Kinetic

##### Energy possessed by moving object

##### “Moving” may be on molecular level – Molecules vibrate when heat increases

### Many types

#### Chemical

#### Thermal

#### Mechanical

#### Electrical

#### Light

#### Nuclear

#### Sound

### All energy can change from one type to another

### In fire behavior – Potential chemical energy of fuel is

#### Converted to thermal energy

#### Released as heat

### Measurement

#### Joules (J) in International System of Units (SI)

#### British thermal unit (Btu) in customary system – Frequently used in fire service

#### 1055 J = 1 Btu

### Chemical, physical changes always involve exchange of energy – Fuel’s potential energy

#### Released during combustion

#### Converted to kinetic energy

### Reactions

#### Exothermic

##### Emit energy as they occur

##### Fire is combustion that releases energy in form of heat, sometimes visible light

#### Endothermic

##### Absorb energy as they occur

##### Converting water from liquid to gas requires input of energy – Important part of controlling, extinguishing some types of fires

## Forms of Ignition

### Process of ignition

#### Fuel is heated – Temperature increases

#### Sufficient heat transfer

##### Causes pyrolysis in solid fuels, vaporization in liquid fuels

##### Releases ignitable vapors or gases

#### Energy necessary for ignition

##### Provided by external source

##### Fuel can be heated until ignites without spark or other source

#### Continues production, ignition of fuel vapors or gases so combustion reaction is sustained

### Forms of ignition

#### Piloted

##### Most common

##### Occurs when mixture of fuel, oxygen encounter external heat source with sufficient heat or thermal energy to start combustion process

#### Autoignition (nonpiloted)

##### Occurs without any external flame or spark to ignite fuel gases or vapors

##### Fuel surface is chemically heated to point at which combustion reaction occurs

##### Autoignition temperature (AIT) – Minimum temperature to which fuel in air must be heated to start self-sustained combustion; always higher than piloted ignition temperature

## Modes of Combustion

### Fire, combustion similar conditions

#### Combustion is chemical reaction – Can occur without fire

#### Fire is one possible result of combustion

### Two modes

#### Nonflaming – Occurs more slowly at lower temperature producing smoldering glow in material’s surface

#### Flaming – Produces visible flame above material’s surface; commonly referred to as fire

### Fire Models – Explain elements of fire, how fires can be extinguished

Instructor Note: The IFSTA validation committee recommends emphasizing to students that both the fire triangle and fire tetrahedron are important fire behavior models to understand. In their opinion, the fire triangle works as an excellent introduction to the idea of the relationship between the elements needed for a fire and how removing one element can work toward extinguishing the fire. The fire tetrahedron should be explained as the industry standard for examining the chemical reaction that fire creates.

#### Fire triangle

##### Oldest, simplest model

##### Three elements necessary – Fuel, oxygen, heat

##### Remove any of elements, fire extinguished

#### Fire tetrahedron

##### Uninhibited chemical chain reaction must be present for fire to occur

##### Created to explain fires involving certain types of substances, types of agents necessary to extinguish

#### Materials that have effect on both ignition, fire development

##### Fuel

##### Heat

##### Oxygen

##### Passive agents – Materials that absorb heat but do not participate actively in combustion reaction

###### Drywall

###### Content of vegetation

###### Relative humidity in air outside of structures

### Nonflaming combustion

#### Occurs when burning is localized on or near fuel’s surface, where in contact with oxygen

#### Examples – Burning charcoal, smoldering wood or fabric

#### Fire triangle used to illustrate

### Flaming combustion

#### Occurs when gaseous fuel mixes with oxygen in correct ratio, is heated to ignition temperature

#### Requires liquid or solid fuels to be vaporized or converted to gas phase through addition of heat

#### Fire tetrahedron reflect conditions required

##### Removing any element interrupts chemical chain reaction, stops flaming combustion

##### Even with elements removed – May continue to smolder

### Products of combustion

#### Produced as fuel burns, changes chemical composition

#### Often described as

##### Heat (energy release, thermal energy)

##### Smoke (new substances)

#### Thermal energy

##### Heats adjacent fuels, makes them more susceptible to ignition

##### Causes fire spread

##### Without adequate protection persons may suffer from burns, damage to respiratory tract, dehydration, heat exhaustion

#### Toxic smoke – Causes most fire deaths

#### Smoke

##### Aerosol comprised of gases, vapor, solid particulates

##### Product of incomplete combustion

##### In structure fires multiple fuels are involved, there is limited air supply

##### Produces wide range of products – Toxic, flammable gases; vapors; particulates

###### Fire gases – Generally colorless

###### Vapor, particulates give smoke color

##### Most components toxic, present significant threat to human life – Effects result of interrelated products present

Instructor Note: Discuss the table on page 215 of the manual. Discuss the types of common products of combustion and their toxic effects with students. In the discussion be sure to review with students the SOPs departments may use to shield them from these effects.

#### Carbon monoxide (CO)

##### Toxic, flammable product of incomplete combustion of organic (carbon-containing) materials

##### Most common product encountered in structure fires

##### Exposure frequently identified as cause of death for both civilians, firefighters

##### Acts as chemical asphyxiant – Binds with hemoglobin in blood preventing cells from distributing oxygen to body

#### Hydrogen cyanide (HCN)

##### Toxic, flammable substance produced by materials containing nitrogen

##### Acts as chemical asphyxiant – Prevents body from using oxygen on cellular level

##### Significant by-product of polyurethane foam – Commonly used in furniture, bedding

#### Carbon dioxide (CO2)

##### Product of complete combustion of organic materials

##### Acts as simply asphyxiant by displacing oxygen

##### Acts as respiratory stimulant – Increasing respiratory rate

#### Irritants in smoke

##### Cause breathing discomfort, inflammation of eyes, respiratory tract, skin

##### Depends on fuels involved

#### Smoke may contain unburned fuel in form of solid, liquid particulates and gases

#### Smoke must be treated with same respect as any other flammable gas – It may burn or explode

#### Firefighters must use SCBA when operating in toxic atmospheres

##### Volume and density reduced during overhaul – But hazard not eliminated

##### Hazardous concentrations above published short-term exposure limits likely to be present during overhaul

##### Hazardous concentrations may be present in areas outside structure

WARNING: Smoke is fuel and is always potentially flammable. Wear full PPE and SCBA anytime you work in smoke.

#### Flame

##### Visible, luminous body of burning gas

##### When mixed with proper amounts of oxygen, flame becomes hotter and less luminous

##### Loss of luminosity caused by more complete combustion

Review Question: How does the science of fire relate to energy, forms of ignition, and modes of combustion?
See pages 208-216 of the manual for answers.

Section II: Thermal Energy (Heat)

# Thermal Energy (Heat)

pp. 216-223	Objective 2 — Describe the impact of thermal energy on heat, temperature, and heat transfer.

  - Thermal Energy (Heat)

### Is energy element of both fire triangle and tetrahedron

### Kinetic energy transferred from high-temperature substance to low-temperature substance

### Always in transit from one location to another

### Thermal kinetic energy needed to release potential chemical energy in fuel

### Vibrates molecules in fuel leading to their breakdown and release of vapors – Vapors can ignite, release thermal energy

## Heat and Temperature

### Temperature is measurement of heat – Average kinetic energy in particles of sample of matter

### Scales used

#### Celsius – Used in metric system

#### Fahrenheit – Used in customary system

## Sources of Thermal Energy

### Chemical energy

#### Most common source of heat in combustion reactions

#### Potential for oxidation exists when any combustible fuel is in contact with oxygen – Process almost always results in thermal energy

#### Self-heating – Form of oxidation

##### Chemical reaction that increases temperature of material without addition of external heat

##### When produced slowly, energy is lost almost as fast as it is generated

##### May be initiated or accelerated by external heat source

#### Spontaneous ignition – To occur material must be heated to autoignition temperature

##### Insulation properties of material immediately surrounding fuel must be such that heat cannot dissipate as fast as being generated

##### Rate of heat production must be great enough to raise temperature of material to autoignition temperature

##### Available air supply in, around material being heated must be adequate to support combustion

#### Rate of oxidation reaction (heat production) increases as more heat is generated, trapped by insulating materials

#### When heat generated exceeds heat being lost, material may reach autoignition temperature

### Electrical energy – Occurs in several ways

#### Resistance heating

##### When electric current flows through conductor, heat is produced

##### Some electrical appliances, designed to make use of resistance heating

##### Other electrical equipment designed to limit resistance heating under normal operating conditions

#### Overcurrent or overload

##### When current flowing through conductor exceeds design limits, may overheat and present ignition hazard

##### Overcurrent or overload is unintended resistance heating

#### Arcing

##### Is high-temperature luminous electric discharge across gap or through medium such as charred insulation

##### May be generated when conductor is separated (such as in electric motor or switch) or by high voltage, static electricity, lightning

#### Sparking

##### When electric arc occurs, luminous (glowing) particles can be formed, spatter away from point of arcing

##### Sparking refers to this spatter, while arc is luminous electric discharge

### Mechanical energy

#### Generated by friction or compression

#### Movement of two surfaces against each other creates heat of friction – Results in heat and/or sparks

#### Heat of compression generated when gas is compressed

## Heat Transfer

### Understanding concept helps firefighters

#### Understand the transfer of heat from initial fuel package to other fuels in, beyond the area of origin affects growth of any fire

#### Use knowledge of heat transfer to estimate size of fire before attacking, to evaluate effectiveness of attack

#### Know that transfer occurs from warmer to cooler objects – Objects at same temperature cannot transfer heat

### Transfer rate

#### Related to temperature differential of bodies, thermal conductivity of solid material involved

#### Greater the temperature differences between bodies, greater transfer rate

#### Heat flux

##### Energy transfer over time per unit of surface area

##### Measured in kilowatts per meter squared (kW/m2)

Instructor Note: Point out the Safety Alert on page 220 of the manual. Discuss what these studies can mean to firefighters wearing PPE. Emphasize that being aware of the condition of your PPE is an important part of situational awareness.

### Conduction

#### Transfer of heat through and between solids

#### Occurs when material is heated as result of direct contact with heat source

#### Results from increased molecular motion; collisions between molecules of substance resulting in transfer of energy through substance

#### More tightly packed molecules are, more readily substance will conduct heat

#### Dependent on

##### Area being heated

##### Temperature difference between heat source and material being heated

##### Thermal conductivity of heated material

Instructor Note: Point out Table 5.3 on page 221 of the manual. Discuss any materials commonly seen in your local jurisdiction. Emphasize how knowing about conductivity can help firefighters during emergency scene operations.

#### Insulating materials slow conduction of heat

##### Disrupt point-to-point transfer of heat or thermal energy

##### Best used in building construction are made of fine particles, fibers with void spaces filled with gas – Gases do not conduct heat well

### Convection

#### Transfer of thermal energy by circulation or movement of fluid (liquid or gas)

#### Usually through movement of hot smoke, fire gases in fire environment

#### May occur in any direction – Generally upward because smoke, fire gases are buoyant

#### Can move laterally

##### Result of differences in pressure

##### Move from areas of high pressure to low pressure

##### Can be from fire area or openings on windward side (higher pressure) to leeward side

#### Will feel increase in temperature when working in flow path – Energy is transferred from gas to skin

Instructor Note: One way to illustrate the concept of convection is to hold your finger in front of a blow dryer on a high heat but low blow setting. The transfer of the heat from the blow dryer to your skin illustrates how working in a flow path will increase temperatures on the fire ground.

### Radiation

#### Transmission of energy as electromagnetic wave, without intervening medium

#### Can become dominant mode of heat transfer when fire grows in size; may affect objects located some distance from fire

#### Influenced by wide range of factors

##### Nature of exposed surfaces

###### Dark materials emit, absorb heat more effectively than lighter color materials

###### Smooth, highly polished surfaces reflect more radiant heat than rough surfaces

##### Distance between heat source and exposed surfaces – Increasing distance reduces effect of radiant heat

##### Temperature difference between heat source and exposed surfaces

###### Temperature difference has major effect on heat transfer through radiation

###### As temperature of heat source increases, radiant energy increases by factor to fourth power

#### Travels in straight line at speed of light

#### Common cause of exposure fires – Fires ignited remote from initial origin

#### Travels through vacuums, air spaces where conduction, convection normally disrupted – Also disrupted by materials that reflect radiated energy

#### Radiant energy sources in fire

##### Flames have high temperature

##### Hot smoke in upper layer

Review Question: What impact does thermal energy have on heat, temperature, and heat transfer?
See pages 216-223 of the manual for answers.

Section III: Fuel

# Fuel

pp. 223-231	Objective 3 — Recognize the physical states of fuel.

## Fuel

### Material or substance oxidized or burned in combustion process

### Scientifically known as reducing agent

### Types

#### Inorganic – Do not contain carbon

##### Hydrogen

##### Magnesium

#### Organic – Containing carbon, other elements

##### Hydrocarbon-based – Gasoline, fuel, oil, plastics

##### Cellulose-based – Wood, paper

### Chemical content influences

#### Heat of combustion

##### Total amount of thermal energy released when specific amount of fuel is oxidized (burned)

##### Usually expressed in kilojoules/gram (kJ/g)

##### Plastics, flammable liquids, flammable gases contain more than wood – Will be encountered in modern construction

#### Heat release rate (HRR)

##### Energy released per unit of time as fuel burns

##### Usually expressed in kilowatts (kW) or megawatts (MW)

##### Dependent on type, quantity, orientation of fuel

##### Directly related to oxygen consumption – Combustion process requires continuous supply to continue

###### Higher HHR with more available oxygen

###### Decreased HHR when ventilation limited

## Gaseous Fuel

### Can be most dangerous of all fuel types – Are already in physical state required for ignition

### Vapor density

#### Describes density of gases in relation to air

#### Air assigned vapor density of 1

##### If gas is less than 1, will rise

##### If gas is greater than 1, will sink

## Liquid Fuel

### Properties

#### Have mass, volume but no definite shape – Except flat surface or shape of container

#### Will not expand to fill all of container

#### When released on ground, will flow downhill, can pool in low areas

### Density compared to water

#### Specific gravity – Ratio of mass of given volume of liquid compared with mass of equal volume of water at same temperature

#### Water assigned specific gravity of 1

##### Liquids with less than 1, will float on surface

##### Liquids with greater than 1, will sink

### Must be vaporized to burn

#### Vaporization – Transformation of liquid to vapor or gaseous state

#### Escaping vapors must be at higher pressure than atmospheric pressure to occur

#### Vapor pressure – Pressure vapors escaping from liquid exert; indicates how easily substance will evaporate or go into air

### Flammable liquids with high vapor pressure present special hazard for firefighters

#### As liquid heats, vapor pressure increases with rate of vaporization

#### Volatility (ease) with which liquid gives off vapor influences how easily it can be ignited

#### Flash point – Minimum temperature at which liquid gives off sufficient vapors to ignite, but not sustain combustion

#### Fire point

##### Temperature at which sufficient vapors are being generated to sustain combustion reaction

##### Commonly used to indicate flammability hazard of liquid fuels

#### Extent to which liquid gives off vapor also influenced by how much surface area is exposed to atmosphere

### Solubility

#### Extent to which substance will mix with water

#### Expressed in qualitative terms or as percentage

#### Miscible materials – Mix in any proportion

#### Hydrocarbon fuels – Do not mix

#### Polar solvents – Readily mix

### Liquids less dense than water difficult to extinguish using water as only extinguishing agent

#### Fuel will not mix with water, adding water may disperse burning liquid instead of extinguishing; potentially spreading fire

#### Should be extinguished with appropriate foam, chemical agent

### Water-soluble liquids will mix with agent, become less effective at extinguishing fire – To avoid use foams specifically designed for polar solvents

## Solid Fuel

### Properties

#### Have definite size, shape

#### May react differently when exposed to heat; some readily melt, others do not

#### When heated – Decompose, release fuel gases, vapors

### Pyrolysis – Chemical decomposition of substance through action of heat

#### Can generate sufficient quantities of burnable vapors to ignite in presence of sufficient oxidizer

#### Must occur to generate flammable vapors required for combustion

##### Begins in wood at temperatures below 400°F (204°C)

##### Occurs much sooner in plastics – No moisture to slow process

Instructor Note: Discuss the table on page 229 of the manual. Discuss the effects of pyrolysis within the temperature zone of your jurisdiction. Emphasize with students how to remain aware of the impact of seasonal changes on these effects.

### Primary consideration for ignition is surface area of fuel in proportion to mass – Called surface-to-mass ratio

#### Example

Instructor Note: Discuss the example of surface-to-mass ratio changes in a large tree provided on page 230 of the manual. Emphasize with students how this concept impacts combustion rates.

#### As surface-to-mass ratio increases – Fuel particles become smaller

#### As surface area increases, more material is exposed to heat, generates combustible pyrolysis products more quickly

### Proximity, orientation of solid fuel relative to source of heat also affects way it burns

Review Question: What are the physical states that fuel can be found in?
See pages 223-231 of the manual for answers.

Section IV: Oxygen

# Oxygen

pp. 231-233	Objective 4 — Explain the relationship between oxygen and life safety.

## Oxygen

### Primary oxidizing agent in most fires – Consists of about 21 percent oxygen

### At normal ambient temperatures (68°F/20°C) – Materials can ignite, burn at oxygen concentrations as low as 14 percent

### When oxygen is limited flaming combustion will diminish – Causing nonflaming combustion

### Impact of ambient temperature

#### Nonflaming can continue at extremely low concentrations when surrounding temperature is relatively low

#### Flaming may continue at lower oxygen concentrations when temperature is high

Instructor Note: Discuss the Safety Alert on page 231 of the manual. Emphasize with students how oxygen concentration impacts both fire behavior and survival.

### When oxygen concentration is higher than normal

#### Materials will burn more intensely, may ignite more readily

#### Some petroleum-based materials will autoignite

#### Materials that do not burn at normal levels will burn

#### Fires may be more difficult to extinguish, present safety hazard to firefighters

### Combustion occurs after fuel has been converted to gaseous state when mixed with oxidizer in proper ratio

#### Range of concentrations called flammable (explosive) range

#### Reported using percent by volume of gas or vapor in air for

##### Lower flammable limit (LFL) – Minimum concentration of fuel vapor, air that supports combustion

##### Upper flammable limit (UFL) – Concentration above which combustion cannot take place

#### Ideal concentration for combustion is within flammable range

Review Question: How do oxygen and life safety relate to one another?
See pages 231-233 of the manual for answers.

Section V: Self-Sustained Chemical Reaction

# Self-Sustained Chemical Reaction

pp. 233-234	Objective 5 — Identify the products of self-sustained chemical reactions.

## Self-Sustained Chemical Reaction

### Example of complex reaction – Combustion of methane, oxygen

#### Complete oxidation of methane results in

##### Production of CO2, water

##### Release of energy in form or heat, light

#### Molecules break apart as flaming combustion occurs – Form free radicals

#### Free radicals combine with oxygen or elements that form fuel material – Produce new substance; leads to more free radical production

#### Production of CO and formaldehyde occurs at various points in process – Both are flammable, toxic

#### Different free radicals, intermediate combustion products created based on type of chemically complex fuels that burn

### Another example – Flaming combustion

#### Sufficient heat causes fuel, oxygen to form free radicals, initiate reaction

#### Fire will burn until fuel or oxygen exhausted or extinguishing agent applied in sufficient quantity to interfere with reaction

#### Chemical flame inhibition occurs when extinguishing agent

##### Interferes with chemical reaction

##### Forms stable product

##### Terminates combustion reaction

Review Question: What products of self-sustained chemical reactions combine to make flammable and toxic substances?
See pages 233-234 of the manual for answers.

Section VI: Stages of Fire Development

# Stages of fire Development

pp. 234-250	Objective 6 — Explain the factors that affect fire development.

Objective 7 — Describe the stages of fire development.

## Stages of Fire Development

### Occur in both unconfined, confined fires

Video clip 00:20

No Sound

Distinct in laboratory simulations, not in exact sequence outside of laboratory

### Single compartment used to illustrate fire progression – Actual conditions in multiple compartments vary widely

### Used at fire scene as guide for what could occur, not a pattern of what will occur every time

Assess changing hazards, conditions at incident – Do not assume fire will follow laboratory pattern

Video clip 00:22
No Sound

## Factors that Affect Fire Development

### Fuel type

#### Affects heat release rate (HRR)

#### Class A fires – High surface-to-mass ratio materials more easily ignited, will burn more quickly

#### Class B fires – Influenced by surface area, type of fuel involved

#### Fires involving single type of fuel rare

##### Modern contents are largely petroleum-based materials – Have higher heat of combustion, produce higher HRRs than wood alone

##### Burning synthetic fuels – Products of combustion contain solid, liquid particulates and unburned gases

#### Compartment fire as result of flammable/combustible gas leak

##### May begin with rapid ignition of gas and explosion

##### If fuel source not controlled may burn at point of release, continue to adjacent combustibles

##### Shutting off fuel source or controlling leak may reduce, eliminate Class B fire – Resulting Class A fire will continue to burn

### Availability and location of additional fuel

#### Building configuration (layout of structure) – Elements can contribute to fire spread or containment

##### Number of stories above or below grade

##### Compartmentation

##### Floor plan

##### Openings between floors

##### Continuous voids or concealed spaces

##### Barriers to fire spread

#### Construction materials – Can add to fuel load

##### Orientation and surface-to-mass ratio of structural materials influence rate, intensity of fire spread

##### Interior finishes can influence fire spread

#### Contents

##### Most readily available fuel source

##### When releasing large amount of heat rapidly – Increases intensity of fire, speed of development

#### Proximity of initial fire to exposed fuel sources

##### Fuels in upper level of adjacent compartments pyrolize more quickly

##### Continuous fuels rapidly spread fire from compartment to compartment

##### Location of fire within building will influence development

### Compartment volume and ceiling height

#### All other factors being equal – Fire in large compartment will develop more slowly than in small compartment

##### Due to greater volume of air, increased distance radiated heat must travel from fire to contents

##### Large volume of air will support development of larger fire

#### High ceiling can make determining extent of fire development difficult

##### Large volume of hot smoke, fire gases can accumulate at ceiling level, while conditions at floor level remain unchanged

##### Do not mistake floor level conditions for actual state of fire development

### Ventilation

#### All buildings exchange air from inside to outside

##### Constructed openings – Windows, doors, passive ventilation devices

##### Leakage through cracks, other gaps in construction

##### Heating, ventilating, and air conditioning (HVAC) system

#### Influenced by size, number, locations of openings, velocity of air being exchanged

#### Can be increased by natural conditions (wind direction, velocity) or assisted by HVAC system

Instructor Note: Discuss the Information Box “Ventilation vs. Tactical Ventilation” on page 239 of the manual. Be sure students understand the distinction between the exchange of air and tactical ventilation practices.

#### Two forms of compartment fires

##### Fuel controlled – Sufficient oxygen available; characteristics, configuration of fuel control development

##### Ventilation controlled

###### Available air supply begins to limit fire development in compartment fire

###### Fire has tendency to grow in direction of ventilation openings

#### Ventilation changes can

##### Alter flow path

##### Create rapid fire development

##### Place firefighters in extreme danger

#### When ventilation controlled – HRR decreases

#### Sudden introduction of air creates rapid increase in HRR

### Thermal properties of the compartment – Can contribute to rapid fire development; make extinguishment difficult, reignition possible

#### Insulation – Contains heat within compartment causing localized increase in temperature, fire growth

#### Heat reflectivity – Increases fire spread through transfer of radiant heat from wall surfaces to adjacent fuel sources

#### Retention – Maintains temperature by absorbing, releasing large amounts of heat slowly

### Ambient conditions

#### High humidity, cold temperatures – Can slow natural movement of smoke

#### Strong winds – Place additional pressure on side of structure, force both smoke, fire out opposite side

#### If window fails or door opens on windward side – Intensity, spread can increase significantly

#### Wind direction, velocity can prevent or assist in ventilation activities

#### Cold temperatures can cause smoke to appear white, give false impression of interior conditions

#### Atmospheric air pressure can cause smoke to remain close to ground obscuring visibility

### Fuel load – Total quantity of combustible contents of building, space, fire area

#### On scene you will only be able to estimate based on knowledge, experience

#### Knowledge of building construction, occupancy types will be essential to determining

Review Question: What different factors can impact fire development?
See pages 234-241 of the manual for answers.

## Incipient Stage

### Starts ignition when three elements of fire triangle come together, combustion process begins

### Fire is small, confined to material first ignited

### Development

#### Largely dependent on characteristics, configuration of fuel involved

#### Air provides oxygen to continue

#### Radiant heat warms adjacent fuel, continues process of pyrolysis

#### Plume of hot gases, flame rises from fire and mixes with cooler air

#### As plume reaches ceiling, begins to spread horizontally across, forming ceiling jet

#### Process of heat transfer begins to increase overall temperature in room

### Characteristics

#### Fire has not yet influenced environment to significant extent

#### Temperature only slightly above ambient

#### Concentration of combustion products low

#### Occupants can safely escape from compartment

#### Fire could be safely extinguished with portable extinguisher, small hoseline

### Essential to recognize that transition from incipient to growth can occur quickly (even in seconds) depending on type, configuration of fuel

## Growth Stage

### As fire transitions

#### Begins to influence environment within compartment

#### Grown large enough for compartment configuration, amount of ventilation to influence it

### Amount of air entrained in plume

#### Affected by location of fuel package in relation to compartment walls – Also impacts amount of cooling taking place

#### Unconfined fires draw from all sides, cools plume of hot gases, reducing flame length, vertical extension

##### Fuel packages in middle of room can entrain air from all sides

##### Fuel packages near walls can only entrain air from three sides

##### Fuel packages in corners can only entrain air from two sides

#### When fuel package not in middle of room – Combustion zone expands vertically, higher plume results

##### Higher plume increases temperature in developing hot-gas layer at ceiling level, increases speed of development

##### Heated surfaces around fire radiate heat back toward burning fuel, increases speed of development

### Thermal layering – Also referred to as heat stratification, thermal balance

#### Tendency of gases to form into layers according to temperature

##### Hottest gases tend to be in upper layer

##### Cooler gases tend to be in lower layer

##### Effects of heat transfer through radiation, convection

##### Radiation from hot gas layer heats interior surfaces of compartment, contents

##### Can be altered by change in ventilation, flow path

#### Pressure increases as volume, temperature do

##### Causes hot gas layer to spread downward within compartment, laterally through openings

##### Cool gas layer pressure lower – Results in inward movement of air from outside at bottom as hot gases exit top

#### Interface of hot, cooler gas layers at opening – Neutral plane

##### Pressure is neutral where layers meet

##### Only exists at openings where hot gases exiting, cooler air entering

#### When possible – Maintain or raise level of hot gas layer above floor

##### Provide more tenable environment for firefighters, trapped occupants

##### Use effective fire control, ventilation to raise position

### Isolated flames

#### May be observed moving though hot gas layer

#### Indicates that portions of layer are within flammable range, there is sufficient temperature to result in ignition

#### Frequently observed prior to more substantial involvement of flammable products of combustion in layer

### Rapid transition

#### Flashover – From growth to fully developed stage

#### Occurs during growth stage in laboratory; may occur at any time conditions are right in uncontrolled situation

#### Does not occur at every fire

##### Development may take alternate path when ventilation limited

##### Ventilation control limits HRR, causing fire to enter decay stage

#### Most fires that develop beyond incipient stage become ventilation controlled

## Fully Developed Stage

### Occurs when all combustible materials in compartment are burning

### Burning fuels in compartment are releasing maximum amount of heat possible for available fuel, oxygen; producing large volumes of fire gases

### Is ventilation controlled because heat release is dependent on compartment openings

#### Provide oxygen, supports ongoing combustion, releases products of combustion

#### Increases in available air supply will result in higher heat release

### Flammable products of combustion

#### Likely to flow from compartment of origin into adjacent compartments or out through openings to exterior of the building

#### Flames will extend out of compartment openings because insufficient oxygen for complete combustion in compartment

Note: If there are limited or no openings in the compartment, it is unlikely that the fire will reach a fully developed stage due to limited ventilation.

## Decay Stage

### Occurs

#### As fuel is consumed

#### If oxygen concentration falls to point that flaming combustion is diminished

### Brings combustion reaction to stop

### Consumption of fuel

#### As fuel is consumed, HRR begins to decline

#### With adequate ventilation, fire becomes fuel controlled

#### Temperature in compartment may remain high even after HRR drops

#### Flammable products of combustion can accumulate within compartment, adjacent spaces

### Limited ventilation

#### HRR declines due to lack of oxygen

#### May maintain extremely high temperature within compartment

#### Pyrolysis can continue

#### Large volume of flammable products of combustion can accumulate

Review Question: What are the stages of fire development?
See pages 241-247 of the manual for answers.

pp. 247-250	Objective 8 — Recognize signs, causes, and effects of rapid fire development.

## Rapid Fire Development

### Responsible for numerous firefighter deaths, injuries

### Protect yourself, crew

#### Recognize indicators

#### Know conditions created by situations

#### Determine best action to take before occurrence

## Flashover

### Combustible materials in compartment, gases produced by pyrolysis ignite almost simultaneously – Full-room involvement

### Typically occurs during growth stage – May occur during fully developed stage

### Environment of room changes from two-layer condition to single well mixed, untenable hot gas condition

### Transition between pre-flashover to post-flashover can occur rapidly

### Conditions during

#### Volume of fire can increase from ¼ to ½ room’s upper volume to fill entire room – Potentially expanding out of any openings

#### Burning gases push out of openings at substantial velocity

### Common elements

#### Transition in fire development – Represents transition from growth stage to fully developed stage

#### Rapidity – Not an instantaneous event, but happens rapidly (often in seconds) to spread complete fire involvement within compartment

#### Compartment – Must be enclosed space such as single room or enclosure

#### Ignition of all exposed surfaces – Virtually all combustible surfaces in enclosed space become ignited

### Factors that determine if fire will progress to flashover

#### Sufficient fuel, heat release rate

#### Ventilation – Must have sufficient oxygen

### Survival rates extremely low – Typically occurs at 1,100oF (600oC) ceiling temperature

### Be aware of indicators

#### Building indicators

##### Can occur in any building

##### Interior configuration, fuel load, thermal properties, ventilation will determine how rapidly

#### Smoke indicators – Rapidly increasing volume, turbulence, darkening color, optical density, lowering of hot gas level

#### Airflow indicators

##### High velocity, turbulence

##### Bi-directional movement with smoke exiting at top of doorway, fresh air moving in at bottom

##### Pulsing air movement

#### Heat indicators

##### Rapidly increasing temperature in compartment

##### Pyrolysis of contents or fuel packages located away from fire

##### Darkened windows

##### Hot surfaces

#### Flame indicators – Isolated flames in hot gas layers or near ceiling

### Rollover also indicator

#### Condition where unburned fire gases at top of compartment ignite, flames propagate through hot gas layer or across ceiling

#### May occur during growth stage

#### Flames may be observed in layer while gases reach combustible temperature

#### Will generally precede, but will not always result in flashover

## Backdraft

### Increase in low-level ventilation prior to upper level ventilation results in explosively rapid combustion of flammable gases

### Occurs in decay stage – In a space containing high concentration of heated flammable gases that lacks sufficient oxygen for flaming combustion

### When potential conditions exist introduction of new source of oxygen will return fire to fully involved state – Can occur with creation of horizontal or vertical opening

### Consider potential for before creating any openings into compartment

### Be aware of indicators

#### Building indicators

##### Fire confined to single compartment or void space

##### Building contents have high heat release rate

#### Smoke indicators

##### Optically dense smoke, light colored or black becoming dense gray-yellow – Color alone is not reliable indicator

##### Neutral plane rising, lowering similar to pulsing or breathing movement

#### Airflow indicators – High velocity, turbulent smoke discharge, sometimes appearing to pulse or breath

#### Heat indicators – High heat, smoke stained windows

#### Flame indicators – Little or no visible flame

### Effects vary depending on factors

#### Volume of smoke

#### Degree of confinement

#### Pressure

#### Speed with which fuel and air are mixed

#### Location where ignition occurs

### Will not always occur immediately after opening made

#### If mix of hot flammable products, air is slow – Unlikely to occur

#### May not occur until air is fully introduced

### Violence depends on extent of confinement – More confined, more violent backdraft will be

## Smoke Explosion

### May occur before or after decay stage

### Occurs as unburned fuel gases contact ignition source

### Cooling smoke can accumulate in other areas, mix with air – If contact ignition source while in flammable range, can result in explosively rapid combustion

### Violent because involve premixed fuel, oxygen

### Smoke generally cool, less than 1,112o F (600o C), located in void spaces connected to fire or in uninvolved areas remote

Review Question: What are the signs and causes of a backdraft?
See pages 248-250 of the manual for answers.

Section VII: Fire Behavior and Fire Fighting Operations

# Fire Behavior and Fire Fighting Operations

pp. 250-253	Objective 9 — Describe the methods through which fire fighting operations can influence fire behavior.

## Temperature Reduction

### Most common method – Cooling with water

#### Enough must be applied to burning fuel to absorb heat generated by combustion

#### Reduces temperature of fuel to point where does not produce sufficient vapor to burn

#### Can extinguish solid, liquid fuels with high flash points

#### Cannot sufficiently reduce vapor production when low flash point flammable liquids, gases involved

#### Most effective method for extinguishment of smoldering fires

### Water can also be used to control burning gases, reduce temperature of hot products of combustion in upper layer

#### Slows pyrolysis process

#### Reduces radiant heat flux from upper layer

#### Reduces potential for flashover

### Water has greatest effect when vaporized into steam

#### Converted to steam at 212oF (100oC)

#### Expands 1,700 times – Avoid creating too much

#### Excess steam

##### Reduces visibility

##### Increase chances for steam burns

##### Disrupts thermal balance

### Control steam production by

#### Using good nozzle technique

#### Applying appropriate amount of water

#### Applying water using most effective form of stream based on conditions

## Fuel Removal

### Simplest method – Allow fire to burn until all fuel consumed; not always desirable

### Best solution may be to allow fire to burn, minimizing groundwater pollution

### Other methods

#### Stop flow of liquid fuel

#### Close valves to stop emission of gaseous fuels

#### Move solid fuels out of path of fire

## Oxygen Exclusion

### Reduces fire’s growth, may totally extinguish over time

### Methods – Do not work if fuel self-oxidizing

#### Flood area with inert gas to displace oxygen

#### Separate oxygen by blanketing fuel with foam

### Closing doors before leaving building can limit air supply, help prevent flashover

## Chemical Flame Inhibition

### Extinguishing agents interrupt combustion reaction, stop flame production

### Effective on gas, liquid fuels

### Do not easily extinguish nonflaming fires – No chemical chain reaction to inhibit

### Not practical for smoldering fires

## Ventilation and Fire Behavior

### Unplanned ventilation – May occur before or after suppression operations start

#### Can result from wind outside structure – Impact

##### Increase pressure inside structure

##### Drive smoke, flames into unburned portions of structure, onto advancing firefighters

##### Upset tactical ventilation efforts

WARNING!: Wind driven conditions can occur in any type of structure. Wind speeds as low as 10 mph (16 kph) can create wind-driven conditions.

#### Can also be result of

##### Occupant action

##### Fire effects on building

##### Action outside of planned, systematic, coordinated tactical ventilation

### Ventilation strategies – Tactical ventilation

#### Planned, systematic, coordinated introduction of air; removal of hot gases, smoke from building

#### Must be coordinated with fire suppression operations to prevent unwanted consequences for hoseline crews

#### Influence on fire behavior based on variety of factors

##### HRR increased in ventilation-controlled fire when ventilation increased

##### Can be as simple as keeping exterior door closed or as complex as performing vertical ventilation

#### Increase in combustion rate when fire is ventilation controlled

WARNING!: Even coordinated tactical ventilation increases the combustion rate in ventilation controlled fires.

Review Question: How can fire fighting operations impact fire behavior?
See pages 250-253 of the manual for answers.

Section VIII: Summary and Review

# SUMMARY AND REVIEW

## Chapter Summary

### You need to understand the combustion process, how fire behaves, and how to select appropriate extinguishing agents.

### Understanding fire behavior can help you recognize developing fire conditions and respond safely to mitigate hazards present in the fire environment.

## Review Questions

### How does the science of fire relate to energy, forms of ignition, and modes of combustion? (208-216)

### What impact does thermal energy have on heat, temperature, and heat transfer? (216-223)

### What are the physical states that fuel can be found in? (223-231)

### How do oxygen and life safety relate to one another? (231-233)

### What products of self-sustained chemical reactions combine to make flammable and toxic substances? (233-234)

### What different factors can impact fire development? (234-241)

### What are the stages of fire development? (241-247)

### What are the signs and causes of a backdraft? (248-250)

### How can fire fighting operations impact fire behavior? (250-253)